Anomaly Detection in Industrial Machine Sounds Using High-Frequency Feature Analysis and Gated Recurrent Unit Networks

ABSTRACT: -

Industrial machinery generates characteristic acoustic patterns that change significantly in the presence of mechanical faults, making sound-based anomaly detection an effective and non-intrusive diagnostic tool. This project presents a deep learning approach for abnormal fan sound detection using a combination of pretrained audio embeddings and sequence modeling. Audio recordings are collected from multiple fan units under normal and abnormal conditions and processed using YAMNet, a pretrained neural network capable of producing high-level acoustic feature representations. These temporal embeddings are then classified using a Bidirectional Gated Recurrent Unit (Bi-GRU) model, enabling the detection of subtle sequential variations in machine sound. To address class imbalance within the dataset, a hybrid resampling strategy combining undersampling and oversampling is employed, and a post-training threshold tuning procedure is applied to maximize abnormal class F1-score. Experimental evaluation demonstrates improved classification performance compared to baseline threshold-based methods, achieving reliable detection of fault conditions in real industrial environments. The proposed system offers a scalable, low-cost, and real-time solution for predictive maintenance in industrial fan systems.

.

| EXSISTING SYSTEM | PROPOSED SYSTEM |
| --- | --- |
| - In the existing system, the authors utilize a deep learning–based method for machine sound anomaly detection primarily focused on sequence modeling using recurrent neural networks. The approach begins by extracting low-level acoustic features such as Mel-Frequency Cepstral Coefficients (MFCC), spectral roll-off, and chroma features from industrial machine sound recordings. These time-series features are then fed into a Gated Recurrent Unit (GRU) network, which is capable of learning temporal dependencies and identifying the gradual changes between normal and abnormal operational states. GRU is chosen in the base paper due to its reduced computational cost and superior convergence behavior over traditional Long Short-Term Memory (LSTM) networks, especially when dealing with continuous audio data with long time dependencies. The existing framework performs classification based on statistical differences between healthy and faulty sound patterns. | - The proposed system introduces a hybrid deep learning architecture using pretrained acoustic embeddings combined with sequential learning to significantly enhance anomaly detection accuracy. Instead of depending solely on handcrafted features, the system uses YAMNet, a neural network pretrained on the Google AudioSet dataset, to extract high-level feature representations from raw audio signals. These embeddings capture complex spectral, temporal, and contextual information present in machine sounds. The resulting sequence of embeddings is then classified using a Bidirectional Gated Recurrent Unit (Bi-GRU), which can analyze sound patterns both forward and backward in time. By integrating pretrained audio feature extraction with sequence modeling, the proposed system improves robustness, accuracy, and fault sensitivity. |
| EXISTING ALGORITHM - Gated Recurrent Units (GRU) | PROPOSED ALGORITHM: - - YAMNet + Bi-GRU (pretrained feature extraction + sequence classifier) |
| ALGORITHM DEFINITION: - A Gated Recurrent Unit (GRU) is a recurrent neural network architecture designed to process sequential data by retaining relevant contextual information through a gating mechanism. GRUs use two gates — the update gate and the reset gate — to control how much information should be remembered or forgotten from previous time steps. Compared to LSTM, GRUs have fewer parameters and are computationally lighter, making them effective for real-time classification of audio signals. In the existing system, GRU acts directly on handcrafted audio features to learn typical signal behavior patterns and detect anomalies in continuous fan sound recordings. | ALGORITHM DEFINITION: - The proposed model combines two powerful components: YAMNet and Bi-GRU. YAMNet is a pretrained deep neural network based on MobileNet architecture trained on millions of audio samples, capable of generating 1024-dimensional embeddings that represent rich and discriminative acoustic features. Bi-GRU expands on traditional GRU by processing temporal information in both directions, enabling stronger understanding of transitions between normal and abnormal fan sounds. This hybrid model takes a sequence of high-level embeddings rather than hand-engineered features, thereby capturing spatial, temporal, and contextual dependencies in industrial sound signals. This design significantly enhances anomaly classification performance by leveraging pretrained general learning and domain-specific sequential learning in a single pipeline. |
| DRAWBACKS: - - Handcrafted feature dependency - Poor generalization - Class imbalance sensitivity - Lack of pretrained knowledge - Limited abnormal precision | ADVANTAGES: - - Higher accuracy - Robust to noise - Better class balance handling - Reduced manual feature engineering - Suitable for real-time deployment |

USER INTERFACE

(Web Page / Desktop Interface)

- Upload .wav file

- Display Prediction

SYSTEM ARCHITECTURE:

FLASK APPLICATION LAYER

Request Handler

Authentication (Temporary)

Routing (API Endpoints)

PROCESSING LAYER

Preprocessing Component

- Load waveform

- Normalize / trim / pad wave

Feature Extraction Component

- YAMNet Embedding (1024-d per frame)

- Feature concatenation

GRU Classification Component

- Bi-directional GRU model

- Output abnormal probability

Threshold Evaluation Component

- Compare probability with threshold

- Generate final label (Normal / Abnormal)

RESULT MODULE

- Prepare final response

- Format output for UI

MODEL STORAGE

- Trained model (.keras)

Fig 4.11: System Architecture

MINIMUMSYSTEM REQUIREMENTS

HARDWARE REQUIREMENTS

- PROCESSOR		:  	Pentium i3 Processor

- RAM			:	4GB DD RAM

- HARD DISK 		:	450 GB

SOFTWARE REQUIREMENTS

- BACK END			: 	PYTHON

- OPERATING SYSTEM 	:  	WINDOWS 10

- IDE				:	Spyder3